# SmartFuseBox PCB Design Specification

Table of Contents

1. Project Overview	2

2. KiCad Project Structure	2

3. ESP32-S3 DevKitC-1 Integration	2

4. Power System	2

5. Relay Output System	3

6. Sensor & IO Ports	3

7. Communication Interfaces	3

8. Monitoring & Auxiliary Features	3

9. Connector Strategy	3

10. 4-Layer PCB Stackup Specification	3

11. Recommended Terminal Blocks	3

Sheets	4

Sheet 1 – Power Management	4

Sheet 2 – ESP32-S3 Module	5

Sheet 3 – Relay Drivers and Outputs	5

Sheet 4 - Sensor Ports and Switch Inputs	6

12. ESP32-S3 Pin Mapping	6

Relay Control Outputs	6

Sensor Input Ports	6

CAN Bus Interface	7

UART Interface	7

Auxiliary Outputs	7

Board Monitoring	7

Reserved / Spare GPIO	7

Power Pins	7

# 1. Project Overview

The SmartFuseBox is a prototype PCB designed for both marine and home automation environments. It is based on the ESP32-S3 DevKitC-1 (N16R8) development board, which is mounted using headers to allow easy replacement. The board distributes 12V power through 8 relay channels (up to 5A each), includes at least 6 sensor input ports, a CAN bus interface, and monitoring features such as an onboard temperature sensor, buzzer, and fan connector. The system is designed for robust operation in demanding environments.

# 2. KiCad Project Structure

The KiCad project is structured as follows:

SmartFuseBox-PCB/
├── SmartFuseBox.kicad_pro
├── SmartFuseBox.kicad_sch ← root sheet (hierarchy only)
├── sheets/
│   ├── Power.kicad_sch
│   ├── ESP32_Module.kicad_sch
│   ├── Relays.kicad_sch
│   ├── Sensors.kicad_sch
│   └── Comms_Monitoring.kicad_sch
├── SmartFuseBox.kicad_pcb
└── gerbers/

# 3. ESP32-S3 DevKitC-1 Integration

The ESP32-S3 DevKitC-1 is mounted via dual-row headers. Essential pins for relays, sensors, CAN, buzzer, and fan are routed appropriately. USB access is maintained for programming. The ESP32 requires 3.3V power and robust grounding with short return paths.

# 4. Power System

The board accepts 12V DC input (up to 40A), distributed across 8 relays. DC-DC converters step down to 5V and 3.3V. Input filtering, reverse polarity protection, and fusing are required. Power planes are used for 12V, 5V, and 3.3V.

# 5. Relay Output System

8 relays with 12V coils (5A each) are driven by MOSFETs or transistors. Flyback diodes suppress voltage spikes. Relay outputs are connected to screw terminals. High-current traces use thick copper and are isolated from logic.

# 6. Sensor & IO Ports

At least 6 sensor ports provide 5V, 3.3V, GND, and two signal pins. All inputs include ESD protection and filtering. Screw terminals are recommended. Ports using GPIO35/36 support analog sensors. GPIO47/48 are digital-only. I2C-capable pairs include GPIO8/9 and GPIO10/11.

# 7. Communication Interfaces

A CAN transceiver interfaces with GPIO38 (TX) and GPIO39 (RX). Proper grounding and shielding are required. Termination resistors should be included as needed.

# 8. Monitoring & Auxiliary Features

Includes onboard temperature sensor (GPIO3), buzzer (GPIO1), and fan control (GPIO2). Optional status LEDs may be added.

# 9. Connector Strategy

Screw terminals are used for all power and sensor connections. ESP32 is socketed via headers. Dedicated connectors are provided for CAN, fan, and buzzer.

# 10. 4-Layer PCB Stackup Specification

Layer 1 — Top Layer: Signal + Low-Current Routing
Layer 2 — Inner Layer 1: Solid Ground Plane
Layer 3 — Inner Layer 2: Power Plane (12V, 5V, 3.3V)
Layer 4 — Bottom Layer: High-Current Relay Routing

# 11. Recommended Terminal Blocks

Use 5.0 mm pitch screw terminal blocks (≥10A rated) for power and relay outputs. Recommended series include:
- Phoenix Contact MKDS 5.08
- Wurth WR-TB series
- KF350-5.0 (generic)
Sensor ports may use 3.5 mm pitch blocks. All terminals should support AWG 12–24 wire sizes.

# Sheets

## Sheet 1 – Power Management

Circuit

| Ref | Part | Value | KiCad Symbol | KiCad Footprint |
| --- | --- | --- | --- | --- |
| J1 | Screw terminal 2-pin | 12V IN | Connector_PinHeader_2.54mm:PinHeader_1x02 | TerminalBlock_Phoenix:TerminalBlock_Phoenix_MPT-2.54mm_2P |
| F1 | Mini blade fuse holder | 20A | Device:Fuse | Fuse:Fuse_BelFuse_0AFF |
| Q1 | IRF9540N | P-ch MOSFET | Device:Q_PMOS_GSD | Package_TO_SOT_THT:TO-220-3_Vertical |
| TVS1 | P6KE15A | TVS 15V | Device:D_TVS | Diode_THT:D_DO-204AC |
| C1 | Electrolytic | 470µF/25V | Device:C | Capacitor_THT:CP_Radial_D10.0mm_P5.00mm |
| U1 | LM2596T-5.0 | Buck regulator | Regulator_Switching:LM2596T-5 | Package_TO_SOT_THT:TO-263-5_Vertical |
| L1 | Power inductor | 100µH/3A | Device:L | Inductor_THT:L_Radial_D8.0mm_P5.00mm |
| D1 | 1N5822 | Schottky 3A | Device:D_Schottky | Diode_THT:D_DO-201AD |
| C2 | Electrolytic | 100µF/10V | Device:C | Capacitor_THT:CP_Radial_D6.3mm_P2.50mm |
| U2 | AMS1117-3.3 | LDO 3.3V | Regulator_Linear:AMS1117-3.3 | Package_TO_SOT_SMD:SOT-223-3_TabPin2 |
| C4/C5 | Tantalum | 10µF/10V | Device:C | Capacitor_SMD:C_0805_2012Metric |

## Sheet 2 – ESP32-S3 Module

Houses the ESP32-S3 DevKitC-1 (N16R8) via two 20-pin female headers. All GPIO signals fan out as named nets to other sheets.

Please see 12. ESP32-S3 Pin Mapping

## Sheet 3 – Relay Drivers and Outputs

8-channel relay control using the ULN2803A Darlington array. Each relay has a status LED and a 3-pin screw terminal output (COM, NO, NC).

| Ref | Part | Value | KiCad Symbol | KiCad Footprint |
| --- | --- | --- | --- | --- |
| U3 | ULN2803ADWR | 8-ch Darlington | Driver_FET:ULN2803A | Package_SO:SOIC-18W_7.5x11.6mm_P1.27mm |
| RL1–RL8 | HRS4H-S-DC12V | 12V coil 10A SPDT | Relay_Electromechanical:G5LE-1 | Relay_THT:Relay_SPDT_HRS4H |
| J_RL1–8 | Screw terminal 3-pin | Relay output | Connector:Screw_Terminal_01x03 | TerminalBlock_Phoenix:TerminalBlock_Phoenix_MPT-2.54mm_3P |
| R_NET1 | Resistor network | 1kΩ×8 | Device:R_Network08 | Resistor_SMD:R_Array_Convex_8x0402 |
| LED1–8 | LED 3mm green | — | Device:LED | LED_THT:LED_D3.0mm |
| R_LED1–8 | Resistor | 470Ω 0402 | Device:R | Resistor_SMD:R_0402 |
| C_RL | Electrolytic | 100µF/25V | Device:C | Capacitor_THT:CP_Radial_D8.0mm_P5.00mm |

## Sheet 4 - Sensor Ports and Switch Inputs

Six 5-pin sensor connectors (5V, 3.3V, GND, SIG1, SIG2) and four 2-pin switch/water-sensor inputs.

SENSOR_PORT_n (5-pin screw terminal):

Pin 1: +5V  (from power rail, 100mA polyfuse per port: PTC_n)

Pin 2: +3.3V (from power rail, 100mA polyfuse per port)

Pin 3: GND

Pin 4: SIG1 → R_PU (10kΩ pullup to 3.3V) → ESP32 GPIO

Pin 5: SIG2 → R_PU (10kΩ pullup to 3.3V) → ESP32 GPIO

ESD protection:

D_ESD: PRTR5V0U2X or similar dual-channel TVS on SIG1/SIG2

# 12. ESP32-S3 Pin Mapping

## Relay Control Outputs

| Relay Channel | ESP32 GPIO | Capabilities | Notes |
| --- | --- | --- | --- |
| Relay 1 | GPIO4 | Digital |  |
| Relay 2 | GPIO5 | Digital |  |
| Relay 3 | GPIO6 | Digital |  |
| Relay 4 | GPIO7 | Digital |  |
| Relay 5 | GPIO15 | Digital |  |
| Relay 6 | GPIO16 | Digital |  |
| Relay 7 | GPIO17 | Digital |  |
| Relay 8 | GPIO18 | Digital |  |

## Sensor Input Ports

| Sensor Port | Signal A | Signal B | Capabilities | Notes |
| --- | --- | --- | --- | --- |
| Sensor 1 | GPIO8 | GPIO9 | Digital / I2C | I2C SDA/SCL capable |
| Sensor 2 | GPIO10 | GPIO11 | Digital / I2C | I2C SDA/SCL capable |
| Sensor 3 | GPIO12 | GPIO13 | Digital |  |
| Sensor 4 | GPIO14 | GPIO21 | Digital |  |
| Sensor 5 | GPIO47 | GPIO48 | Digital only | No ADC support |
| Sensor 6 | GPIO35 | GPIO36 | Analog only | ADC input only |

## CAN Bus Interface

| Function | ESP32 GPIO | Capabilities | Notes |
| --- | --- | --- | --- |
| CAN TX | GPIO38 | Digital |  |
| CAN RX | GPIO39 | Digital |  |

## UART Interface

| Function | ESP32 GPIO | Capabilities | Notes |
| --- | --- | --- | --- |
| UART TX | GPIO43 | USB-CDC | Avoid if using native USB |
| UART RX | GPIO44 | USB-CDC | Avoid if using native USB |

⚠ If USB-CDC is used, reassign UART to GPIO1/2 or GPIO17/18.

## Auxiliary Outputs

| Function | ESP32 GPIO | Capabilities | Notes |
| --- | --- | --- | --- |
| Buzzer | GPIO1 | Digital | Can be used for UART TX if needed |
| Cooling Fan | GPIO2 | Digital | Can be used for UART RX if needed |

## Board Monitoring

| Function | ESP32 GPIO | Capabilities | Notes |
| --- | --- | --- | --- |
| Temperature Sensor | GPIO3 | Analog | ADC input |

## Reserved / Spare GPIO

| GPIO | Capabilities | Notes |
| --- | --- | --- |
| GPIO40 | Digital |  |
| GPIO41 | Digital |  |
| GPIO42 | Digital |  |
| GPIO45 | Input only | Do not use for output |
| GPIO46 | Input only | Do not use for output |

## Power Pins

| Function | Connection |
| --- | --- |
| 5V | 5V regulator output |
| 3.3V | 3.3V regulator output |
| GND | Ground plane |